IoT AC Power Dimmer

with ESP32 + ESP-NOW Wireless Remote

Project Design Document  |  March 2026

Project Summary A two-board wireless AC dimmer using ESP32 microcontrollers on both the dimmer and remote. The dimmer board connects to 230V mains and uses phase-angle TRIAC control. The remote runs on a 3.7V LiPo battery with USB-C charging via TP4056. Both boards communicate over ESP-NOW..

| Parameter | Dimmer Board | Remote Board |
| --- | --- | --- |
| MCU | ESP32-WROOM-32 (240MHz dual-core) | ESP32-WROOM-32 (240MHz dual-core) |
| Wireless | ESP-NOW RX + WiFi (simultaneous) | ESP-NOW TX only (on wakeup) |
| Power supply | HLK-PM03 (230V→5V direct) | 3.7V LiPo + TP4056 |
| AC control | TLP281-4 + MOC3021 + BT138 | None |
| Max load | ~2000W (BT138 12A) | — |
| Phase timing | hw_timer_t Core 0 (hardware ISR) | — |
| Sleep current | ~120mA (PSU dominant) | ~5µA deep sleep |
| Battery life | — | ~4-6 months (500mAh LiPo) |
| Programming | USB-C via CP2102 on-board | USB-C via CP2102 on-board |
| Future expansion | MQTT over WiFi, OTA updates | BLE, display add-on possible |

# 1. Project Overview

We will make both the ESPs communicate with each other and with the remote board’s instruction, the dimmer board will adjust the AC Supply.

# 2. System Architecture

## 2.1 Dimmer board block diagram

| Block | Component | Function |
| --- | --- | --- |
| AC IN | MKDSN 5.08mm screw terminal | Live + Neutral mains input |
| Isolated PSU | HLK-PM03 | 230V AC to 3.3V/900mA, isolated. No LDO needed. |
| ZC detector | TLP281-4 + R1 10kΩ + D1 1N4007 + R2 10kΩ pull-up | Detects mains zero crossings, outputs 3.3V pulses to GPIO34 |
| Gate drive | MOC3021 + R3 330Ω | Optoisolated TRIAC gate trigger from GPIO25 |
| Power switch | BT138-600 TRIAC + R4 470Ω gate + R5+C5 snubber | Controls AC power to load |
| MCU | ESP32-WROOM-32 | Core 0: phase timing. Core 1: WiFi + ESP-NOW + MQTT |
| Programming | CP2102 USB-Serial + USB-C + auto-reset circuit | Flash firmware without ST-Link |
| AC OUT | MKDSN 5.08mm screw terminal | Output to load |
| PCB slot | Routed cutout in FR4 | Galvanic isolation under TLP281-4 and MOC3021 |

## 2.2 Remote board block diagram

| Block | Component | Function |
| --- | --- | --- |
| Battery | LiPo 500mAh 3.7V (JST-PH) | Main power source, rechargeable |
| Charger | TP4056 module | USB-C to LiPo 1A |
| LDO | ME6211 3.3V SOT-23-5 | 3.7V LiPo to regulated 3.3V for ESP32 |
| MCU | ESP32-WROOM-32 | Deep sleep, ext1 wakeup on buttons, ESP-NOW TX |
| Buttons | 3x tactile 6x6mm + 100kΩ pull-up + 100nF debounce | UP / DOWN / ON-OFF |
| Programming | CP2102 USB-Serial + USB-C | Firmware flashing |
| Wakeup pins | GPIO39, GPIO34, GPIO35 (RTC GPIOs) | Only RTC GPIOs survive deep sleep for ext1 wakeup |

# 3. Circuit Diagrams

## 3.1 Dimmer board

The HLK-PM03 outputs 3.3V directly — no AMS1117 LDO is needed, simplifying the board significantly compared to v1.0. The TLP281-4 ZC detector feeds GPIO34 (input-only RTC GPIO). The MOC3021 gate drive receives its signal from GPIO25 via R3 330Ohm. The BT138 TRIAC fires after a delay set by hw_timer_t on Core 0. The CP2102 USB-Serial chip enables programming directly from USB-C without any external programmer.

Figure 1 — ESP32 Dimmer Board. Red rails = 230V AC. Green dashed = 3.3V bus. Note: HLK-PM03 directly outputs 3.3V — no LDO between PSU and ESP32.

## 3.2 Remote board

The LiPo battery feeds the TP4056 charger (which also accepts USB-C input) and the ME6211 LDO simultaneously. The ME6211 regulates 3.7V to a clean 3.3V for the ESP32. Buttons use 100kOhm pull-ups (not 10kOhm) to minimise leakage current during deep sleep. All three button GPIOs (39, 34, 35) are RTC-capable and used with esp_sleep_enable_ext1_wakeup(). The CP2102 provides USB-C programming.

Figure 2 — ESP32 Remote Board. LiPo + TP4056 + ME6211 power chain. All button pins are RTC GPIOs for deep sleep ext1 wakeup.

# 4. Pin Mapping

## 4.1 Dimmer board — ESP32 GPIO assignments

| GPIO | Signal | Direction | Notes |
| --- | --- | --- | --- |
| GPIO34 | ZC detect (TLP281-4 output) | Input only | Input-only RTC GPIO. No internal pull-up. Use external 10kOhm pull-up to 3.3V. |
| GPIO25 | TRIAC gate (MOC3021 LED via R3) | Output | DAC-capable but used as digital. hw_timer_t fires 100us pulse here. |

## 4.2 Remote board — ESP32 GPIO assignments

| GPIO | Signal | Direction | Notes |
| --- | --- | --- | --- |
| GPIO39 | BTN UP | Input only | RTC GPIO. Input-only. External 100kOhm pull-up to 3.3V. ext1 wakeup source. |
| GPIO34 | BTN DOWN | Input only | RTC GPIO. Input-only. External 100kOhm pull-up. ext1 wakeup source. |
| GPIO35 | BTN ON/OFF | Input only | RTC GPIO. Input-only. External 100kOhm pull-up. ext1 wakeup source. |

Document prepared by Harsh  |  ESP32 IoT Dimmer Project v2.0  |  March 2026